#include <Arduino.h>

#include <ESP8266WiFi.h>

#include <ESP8266WiFiMulti.h>

#include <ESP8266HTTPClient.h>

#include <WiFiClient.h>

// WiFi credentials

const char* ssid = "Redmi Note 11T 5G";     //  WiFi SSID

const char* password = "123456789";  // WiFi Password

ESP8266WiFiMulti WiFiMulti;

void setup() {

pinMode(D2, OUTPUT);      // LED/Buzzer connected to D2

Serial.begin(9600);

// Connect to WiFi

WiFi.mode(WIFI_STA);

WiFiMulti.addAP(ssid, password);

Serial.println("Connecting to WiFi...");

while (WiFiMulti.run() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

void loop() {

if (WiFiMulti.run() == WL_CONNECTED) {

WiFiClient client;

HTTPClient http;

// PC's LAN IP

String url = "http://10.124.223.6/IoT_Practicals/fetch_get.php?name=LEDON";

//String url = "http://172.20.10.5/IoT_Practicals/fetch_get.php?name=LEDON";

Serial.println("[HTTP] Requesting: " + url);

http.begin(client, url);  // Start connection

int httpCode = http.GET();  // Send GET request

if (httpCode > 0) {

Serial.printf("[HTTP] Response code: %d\n", httpCode);

if (httpCode == 200) {

String payload = http.getString();

Serial.println("Server Response: " + payload);

if (payload == "ON") {

digitalWrite(D2, HIGH);

Serial.println("LED/Buzzer ON");

delay(2000);

digitalWrite(D2,LOW);

} else if (payload == "OFF") {

digitalWrite(D2,LOW);

Serial.println("LED/Buzzer OFF");

}

}

} else {

Serial.printf("[HTTP] GET failed, error: %s\n",

http.errorToString(httpCode).c_str());

}

http.end();  // Close connection

}

delay(2000);  // Wait 2 seconds before next request

}